Name: _______________________

Water Level Activity

Setup and Materials:

- Break up into 5 groups.

- You will receive one water level sensor which contains an OLED Display Screen, a Boron with a battery, and a distance sensor.

- You will plug the battery into the Boron and you should see the Boron start flashing and your LED screen say “Hello, MSA!”

Activity Steps:

- Fill out the table with 10 different distances that are between 50 cm and 300 cm. For each measurement you will follow these steps:

  - Using a measuring stick, measure a specific distance from a wall. Write that number under “Actual (cm)”. This is the actual distance to the wall.

  - You will then hold your sensor at that same distance and wait 15-20 seconds and then write down the “ADC count” you see on the screen. Waiting allows the distance sensor to adjust. Make sure that there are no large objects near the sensor beam that it may pick up.

  - Once you have your “ADC Count” for your measurement, multiply by .25 to receive your “Calculated Distance (cm)”. Write this in your table.

  - Considerations! Make sure to wait 15 seconds and that you are holding the sensor still and perpendicular to the wall. It will interfere with the reading if your sensor is pointing at an angle or shaking.

- Alternating roles!

  - Measurer: Handle measuring exact distances from the wall

  - Sensor Handler: Makes sure that the sensor is perpendicular, straight, and still while waiting for a sensor read.

  - Recorder: Ensures that exact values are noted on the table and shared with other group members.

  - Supervisor: VERY IMPORTANT! Make sure all directions are followed.

- Complete the table on the other side.

Water Level Activity Table

| Attempt | Actual Distance (cm) | ADC Count from OLED Screen | Conversion Factor | Calculated Distance (cm) |
| --- | --- | --- | --- | --- |
| Example | 157 cm | 640 | x .25 | 160 cm |
| 1 |  |  | x .25 |  |
| 2 |  |  | x .25 |  |
| 3 |  |  | x .25 |  |
| 4 |  |  | x .25 |  |
| 5 |  |  | x .25 |  |
| 6 |  |  | x .25 |  |
| 7 |  |  | x .25 |  |
| 8 |  |  | x .25 |  |
| 9 |  |  | x .25 |  |
| 10 |  |  | x. 25 |  |