Stormwater Pond Analysis

| Assumptions: - The retention pond is made up of upside-down trapezoids - The depth is 4 feet in the two main pools - The depth of the fore bay (3) is 3 feet - The slopes are 3H:1V | Calculations: - h = depth - Ab = Area of bottom - At = Area of top 1 ft3 = 7.48 gallons of water |
| --- | --- |

Example Work for Arbitrary Main Pond:

At = (50 ft x 60 ft) = 3000 ft2       <----- 50 ft and 60 ft would have been measured by you!

Based on 3H:1V, we know that for every 1 vertical foot (depth) it is 3 feet horizontal (H). Therefore, if our pool is 4 feet deep, the slope of the pool is about (4 x 3) 12 feet long. This means that the length and width of the bottom of the pool are both 12 feet less than At.

Ab = (50 ft –12 ft) x (60 ft –12 ft) = (38 ft x 48 ft) = 1824 ft2.

V =

43(1824 + 3000+1–√824×3000)431824 + 3000+1824×3000

= 9,550 ft3

9,550 ft3 x (7.48 gallons / 1 ft3) = 71,441 gallons of water

Your work:

How many gallons of water can your volume hold? What is the total volume of water?

Based on the first lesson, what is impacting how much water enters this pond?